# Jumpers and Connectors

## What kind of CPU are you using ?

This section contains the detail descriptions of Intel, AMD, Cyrix, and UMC processors.  Before you setting the jumpers, you should read this section to know the CPU voltage, CPU clock, etc.

- Intel CPU:

- Intel486 SX SL Enhance processor

| A80486SX-xx: FFFFFFFF: &E5V1XSX###: | Product No. - Frequency (either 25 or 33 MHz) FPO# &E=SL Enhance; 5 volt; 1X clock; spec# |
| --- | --- |

- IntelSX2 SL Enhance processor

| A80486SX2-50: FFFFFFFF: &E5V1XSX###: | Product No. - Frequency (50MHz) FPO# &E=SL Enhance; 5 volt; 1X clock; spec# |
| --- | --- |

- Intel486 DX SL Enhance processor

| A80486DX-33: FFFFFFFF: &E5V1XSX###: | Product No. - Frequency (33 MHz) FPO# &E=SL Enhance; 5 volt; 1X clock; spec# |
| --- | --- |

- IntelDX2 SL Enhance processor

| A80486DX2-xx: FFFFFFFF: &E5V1XSX###: | Product No. - Frequency (either 50 or 66 MHz) FPO# &E=SL Enhance; 5 volt; 1X clock; spec# |
| --- | --- |

- IntelDX2 SL Enhance L1-WB processor (P24D)

| A80486DX2-xx: FFFFFFFF: &EW5V1XSX###: | Product No. - Frequency (either 50 or 66 MHz) FPO# &E=SL Enhance; W=L1-Write Back; 5 volt; 1X clock; SX954/SX955 |
| --- | --- |

- IntelDX4 SL Enhance processor (P24C)

| A80486DX4-xx: FFFFFFFF: &E3VOLTSX###: | Product No. - Frequency (either 75 or 100 MHz) FPO# &E=SL Enhance; 3.3 volt; New S-spec# |
| --- | --- |

- IntelDX2 OverDrive processor

| DX2ODPR-xx: FFFFFFFF: ##### v4.0: | Product No. - Frequency (either 50 or 66 MHz) FPO# SL Enhance; 5 volt |
| --- | --- |

- IntelDX4 OverDrive processor

| DX4ODPR-xx: FFFFFFFF: ##### v1.x: | Product No. - Frequency (either 75 or 100 MHz) FPO# SL Enhance; 5 volt |
| --- | --- |

- IntelDX/SX/DX2/SX2 Non-SL Enhance processor

| A80486xx-xx: FFFFFFFF: SX###: | Product No. - Frequency (25/33/50/66 MHz) FPO# 5 volt |
| --- | --- |

- AMD CPU:

- Cyrix CPU:

| CPU Type | CxDX4-100GP4 | DxDX4-100GP |
| --- | --- | --- |
| Surface Mark | Cx486DX4-100GP DX4-P/O 3.45V | Cx486DX4-100GP 3.45V |

| Part Number | CPU Frequency | Nominal Voltage | Voltage range |
| --- | --- | --- | --- |
| Cx486DX2-V50 | 50 MHz | 3.3 or 3.6 Volts | 3.15 - 3.75 Volts |
| Cx486DX2-V66 | 66 MHz | 3.6 Volts | 3.45 - 3.75 Volts |
| Cx486DX2-V80 | 80 MHz | 4.0 Volts | 3.8 - 4.2 Volts |
| CxDX4-100GP | 100 MHz | 3.45 Volts | 3.45 - 3.6 Volts |
| CxDX4-100GP4 | 100 MHz | 3.45 Volts | 3.45 - 3.6 Volts |
| Cx5X86-100/120GP | 100/120 MHz | 3.45 Volts | 3.45 - 3.6 Volts |

|  | SGS, TI, and IBM CPU are compatible with Cyrix CPU. Please contact with your CPU vendor for the detailed information. |
| --- | --- |

|  | If there are no "V" (Voltage) on the surface of your AMD, Cyrix, or UMC CPU, the meaning is 5V core CPU. |
| --- | --- |

## Setting the Jumpers

The table below summarizes the functions and jumper settings of each jumper on the UP8812 AIO.  Please refer to the next section for the graphic descriptions.

| Function |  | Jumper Settings |
| --- | --- | --- |
| CPU Type | Intel 80486DX/DX2 AMD Am486DX/DX2 Am486DX4 NV8T | JP9 open JP10 open JP12 short 1-2 JP13 short 1-2 JP15 open JP16 open JP17 open JP18 short 1-2 JP21 open JP23 open JP25 open JP26 short 1-2 JP27 open JP28 short 1-2, 3-4 JP29 open |
|  | Intel 80486DX/DX2/DX4 (SL Enhance, L1 Write-Through) ¶ see the page 3-7 Overdrive DX2/DX4 ODPR | JP9 short 2-3 JP10 open JP12 short 1-2 JP13 short 1-2 JP15 open JP16 open JP17 short 1-2 JP18 short 2-3 JP21 open JP23 open JP25 short 1-2 JP26 short 1-2 JP27 open JP28 short 1-2, 3-4 JP29 short 2-3, 4-5 |

Continued .....

| Function |  | Jumper Settings |
| --- | --- | --- |
| CPU Type | Intel 80486SX (PGA) | JP9 open JP10 open JP12 short 1-2 JP13 short 1-2 JP15 open JP16 open JP17 open JP18 short 1-2 JP21 open JP23 open JP25 open JP26 open JP27 open JP28 short 2-3 JP29 open |
|  | Intel 80486SX (SL Enhance) | JP9 open JP10 open JP12 short 1-2 JP13 short 1-2 JP15 open JP16 open JP17 short 1-2 JP18 short 2-3 JP21 open JP23 open JP25 short 1-2 JP26 open JP27 open JP28 short 2-3 JP29 short 2-3, 4-5 |
|  | Intel PD5V (P24T) | JP9 open JP10 open JP12 short 1-2 JP13 short 1-2 JP15 short 1-2 JP16 open JP17 short 1-2 JP18 short 1-2 JP21 open JP23 short JP25 short 1-2 JP26 short 2-3 JP27 short 1-2 JP28 short 1-2, 3-4 JP29 short 2-3, 4-5 |

Continued .....

| Function |  | Jumper Settings |
| --- | --- | --- |
| CPU Type | Intel 80486DX/DX2/DX4 (SL Enhance, L1 Write-Back) ¶ see the page 3-7 AMD Am486DX2/DX4 SV8B (SL Enhance, L1 Write-Back) Am5x86-P75 Cyrix Cx486DX4-100GP4 Cx5x86 | JP9 short 1-2 JP10 open JP12 short 1-2 JP13 short 1-2 JP15 open JP16 open JP17 short 1-2 JP18 short 2-3 JP21 open short (for AMD Am5x86-P75) JP23 open JP25 short 1-2, 3-4 JP26 short 1-2 JP27 open JP28 short 1-2, 3-4 JP29 short 2-3, 4-5 |
|  | Cyrix Cx486S (M6) | JP9 open JP10 open JP12 short 1-2 JP13 short 2-3 JP15 short 2-3 JP16 short 2-3 JP17 short 1-2 JP18 short 2-3 JP21 open JP23 open JP25 short 2-3 JP26 open JP27 short 2-3 JP28 short 2-3 JP29 short 1-2, 3-4 |

Continued .....

| Function |  | Jumper Settings |
| --- | --- | --- |
| CPU Type | Cyrix Cx486DX/DX2/DX4 (M7) | JP9 open JP10 open JP12 short 1-2 JP13 short 2-3 JP15 short 2-3 JP16 short 2-3 JP17 short 1-2 JP18 short 2-3 JP21 open JP23 open JP25 short 2-3 JP26 short 1-2 JP27 short 2-3 JP28 short 1-2, 3-4 JP29 short 1-2, 3-4 |
| Intel 80486DX4 | 2X External Clock | JP21 short |
| Internal Clock | 3X External Clock | JP21 open |
| CPU Clock Select | 25 MHz | JP5 short 1-2 |
|  | 33 MHz | JP5 short 1-2, 3-4, 5-6 |
|  | 40 MHz | JP5 short 1-2, 3-4 |
|  | 50 MHz | JP5 short 5-6 |
| CPU Voltage | 5V | JP30 open JP32 short 1-2 JP33 short 1-2 |
|  | 3.3V | JP30 short 1-2 JP32 short 2-3 JP33 short 2-3 |
|  | 3.45V | JP30 short 3-4 JP32 short 2-3 JP33 short 2-3 |
|  | 3.6V | JP30 short 5-6 JP32 short 2-3 JP33 short 2-3 |
|  | 4.0V | JP30 short 7-8 JP32 short 2-3 JP33 short 2-3 |

Continued .....

| Function |  | Jumper Settings |
| --- | --- | --- |
| External Cache | 128K (32Kx8 SRAMs in one bank) | JP6 open JP7 open JP8 open JP14 open JP19 open JP24 short 2-3 |
|  | 256K (32Kx8 SRAMs in two banks) | JP6 short JP7 open JP8 open JP14 open JP19 open JP24 short 1-2 |
|  | 256K (64Kx8 SRAMs in one bank) | JP6 short JP7 open JP8 open JP14 open JP19 short 1-2 JP24 short 2-3 |
|  | 512K (64Kx8 SRAMs in two banks) | JP6 short JP7 short JP8 open JP14 open JP19 short 2-3 JP24 short 1-2 |
| External Cache | 512K (128Kx8 SRAMs in one bank) | JP6 short JP7 short JP8 open JP14 short 1-2 JP19 short 1-2 JP24 short 2-3 |
|  | 1M (128Kx8 SRAMs in two banks) | JP6 short JP7 short JP8 short JP14 short 2-3 JP19 short 2-3 JP24 short 1-2 |
| On-board Multi-I/O | Enabled | JP3 short 1-2 |
|  | Disabled | JP3 short 2-3 |

Continued .....

| Function |  | Jumper Settings |
| --- | --- | --- |
| ROM Type of BIOS | EPROM | JP1 open |
|  | Programming Flash ROM with +5 Voltage | JP1 short 1-2 |
|  | Programming Flash ROM with +12 Voltage | JP1 short 2-3 |

Table 3-1.   Jumper Settings

## Graphic Descriptions of Jumper Settings

### CPU Type

- Intel 80486DX/DX2, AMD Am486DX/DX2, Am486DX4 NV8T

- Intel 80486DX/DX2/DX4 (SL Enhance, L1 Write-Through),
       Overdrive DX2/DX4 ODPR

- Intel 80486SX (PGA)

- Intel 80486SX (SL Enhance)

- Intel PD5V (P24T)

- Intel 80486DX/DX2/DX4 (SL Enhance, L1 Write-Back);
AMD Am486DX2/DX4 SV8B (SL Enhance, L1 Write-Back),
Am5x86-P75; Cyrix Cx486DX4-100GP4, Cx5x86

- Cyrix Cx486S (M6)

- Cyrix Cx486DX/DX2/DX4 (M7)

### External Cache

- 128K (32Kx8 SRAMs in one bank)

- 256K (32Kx8 SRAMs in two banks)

- 256K (64Kx8 SRAMs in one bank)

- 512K (64Kx8 SRAMs in two banks)

- 512K (128Kx8 SRAMs in one bank)

- 1M (128Kx8 SRAMs in two banks)

## Connectors

The following table lists the connectors located on the UP8812 AIO.  They are used to connect with some peripheral devices to enhance the operating performance of the system.

Please refer to the mainboard layout figure on next page for their positions.

| Connector | Function |
| --- | --- |
| J1 | PS/2 Mouse Connector (optional) |
| J2 | PS/2 Keyboard Connector (optional) |
| J3 | Serial Port 1 (COM1) |
| J4 | Serial Port 2 (COM2) |
| J5 | Parallel Port |
| J9 | Secondary IDE Connector |
| J10 | Primary IDE Connector |
| J11 | Floppy Connector |
| J13 | HDD LED Connector |
| J14 | 5-pin PS/2 Mouse Connector (optional) |
| JP31 |  |
| KB1 | AT Keyboard Connector |
| PS1 | Power Connector |

Table 3-2.   Connectors

## Board Layout

Figure 3-1.   UP8812 AIO Main board Layout